# 08_recommendation

# §08 — Recommendation

Revision: 1 Last modified: 2026-05-22T07:20:02Z Authority: vasic-digital tmux project (research-only) Maintainer: milosvasic Scope: Synthesis — should we port the vasic-digital tmux project to Termux on Android?

## TL;DR

Conditional YES. A Termux port is technically feasible and would deliver real operator value. Recommended sequencing: do NOT port now; defer until v1.0.9 lands and the test harness for nezha-Linux is hardened. Then add Termux as the third platform (§11.4.81 cross-platform-parity branch) in a focused PWU. Effort: medium (≈ 2-3 weeks of subagent-driven work). Maintenance: medium-ongoing.

## Decision matrix

| Dimension | Assessment | Notes |
| --- | --- | --- |
| Build complexity | LOW-MEDIUM | tmux + libevent + jemalloc all build cleanly on bionic (Termux's own package proves it). Go cross-compile is well-trodden territory. setup.sh extension is ~50 LOC. |
| Isolation primitives | MEDIUM | Lose cgroup features. Gain RLIMIT_AS enforcement vs macOS. Honest-gap citation table is clean. |
| End-user UX | MEDIUM | Touch keyboard is awkward but workable (Vol-Down emulates Ctrl, extra-keys row). Background suspension is the biggest pain point — requires operator wake-lock dance + battery-opt disable. |
| Test coverage | MEDIUM | SSH-driven harness is straightforward (same shape as nezha bridge). New failure-mode classes (Doze, Phantom Process Killer, lmkd) require explicit topology detection. ~200 LOC new test scaffolding. |
| Maintenance burden | MEDIUM-ONGOING | Android version drift every 12 months. Termux package cadence is currently slower than 2019-2022 peak. Need at least one phone in regular CI rotation. |
| Operator demand | UNKNOWN | This research was prompted by a "could we" question, not a count of requests. Validation needed: ask operators whether they'd actually use it. |

## Recommendation rationale

A Termux port is not technically expensive. The hard parts of tmx (per-session isolation, anti-bluff testing, multi-platform parity) are already designed to extend cleanly via §11.4.81. The new mechanical work is:

- ~50 LOC in install_deps.sh + setup.sh for Termux detection and package install.

- A Termux branch in tmx.template reusing the existing rlimit wrapper.

- A Port 8022 flag in tmx-ssh-install.sh.

- A documented operator-prep checklist (wake-lock, battery-opt, Phantom killer).

- ~200 LOC of test-harness adaptation per §07.

- Documentation: new docs/guides/tmx-on-termux.md + per-script companion guides.

What does NOT need work: the Go state daemon, the shell-init script, the SSH dispatcher, tmux itself, jemalloc preload, the project's verify/test/Challenge infrastructure.

The expensive part is maintenance: we'd commit to running Termux tests on every release, keeping pace with Android API changes, and supporting end users on a less-predictable OS.

## Conditions for proceeding

The conditional "yes" depends on:

- v1.0.9 ships clean on Linux + macOS first. Adding a third platform mid-feature-development is §11.4.42-iteration-discipline violation territory.

- Operator demand is validated. A 1-question Issues.md poll: "Would you use tmx on your Android phone via Termux?" If <30% yes, defer the port until demand grows.

- At least one test device dedicated. Either a developer's spare phone or a 2-3 year old phone we keep plugged into the test bench. Termux on an emulator is insufficient for the §06 WILL-DEGRADE items.

- Tailscale or equivalent established for remote access. Without it, dev-loop friction (laptop ↔ phone test cycle) is prohibitive on cellular networks.

- One operator volunteers to be the Termux-using-end-user beta tester. §11.4 covenant demands end-user-validation; we need a real end user who actually uses tmux on Android.

If all five hold: ship a v1.1.x feature release adding Termux support. If any fail: defer.

## What would the v1.1.x scope look like?

| PWU | Scope | Estimate |
| --- | --- | --- |
| T1 | install_deps.sh + setup.sh Termux branch | 2 days |
| T2 | tmx.template Termux dispatch + RLIMIT_AS branch | 2 days |
| T3 | tmx-ssh-install.sh --port 8022 autodetect | 1 day |
| T4 | tmx doctor Termux-environment probe (wake-lock, battery-opt, Phantom killer) | 2 days |
| T5 | Test harness: scripts/test_termux.sh + per-test Termux branches | 4 days |
| T6 | Pre-build gates + paired §1.1 mutations for Termux | 2 days |
| T7 | Docs: docs/guides/tmx-on-termux.md, operator setup checklist, troubleshooting | 2 days |
| T8 | First real-device shake-out cycle (10× full retest per §11.4.50) | 1 day |
| T9 | §11.4.40 full-suite retest on Linux+macOS+Termux before tagging | 1 day |
|  | Total | ~17 working days |

That fits comfortably into a 3-week iteration with subagent parallelism per §11.4.58 + §11.4.70. PWUs T1-T3 are parallel; T4-T5 are parallel; T6-T7 are parallel; T8-T9 serial.

## What we do NOT recommend

- A "minimal viable port" that skips lmkd / Doze handling. That is exactly the §11.4 "tests green but feature broken for end user" failure mode the covenant exists to prevent. If we port, we port with the operator-prep checklist + the tmx doctor probe + the SKIP-with-reason vocabulary for unavoidable Android killers. Otherwise we ship a feature that "works on my screen-on dev phone" and breaks for everyone else.

- A Play Store distribution. Termux itself isn't on Play Store (frozen 2022). We have no business trying to ship a tmux wrapper there.

- A rooted-cgroup-v2 default path. Document the rooted opt-in for power users, but the supported default is unrooted Termux with rlimit.

- Cross-host state sync. v1.0.9 explicitly excludes this (spec §3 non-goal). Adding it to support "my work session on phone matches my work session on laptop" doubles the design surface. Stay focused.

## Open questions for operator approval

Per §11.4.66 (blocker-resolution-interactive-clarification), the decisions that ultimately need operator sign-off:

- Go / no-go on the v1.1.x port itself. This document recommends conditional-yes with the five conditions above. Operator may say "go ahead anyway" or "defer indefinitely."

- Default port 8022 vs alternate. 8022 is Termux convention; we recommend keeping it.

- Wake-lock policy. Default to operator-runs-it-themselves (notice), or wrapper auto-acquires?

- Whether to publish a Termux-targeted recipe via termux-packages upstream. Path (c) in §02 — currently not recommended, but operator may differ.

- CI investment. Buy a dedicated test phone (~$300-500) and integrate it into the nezha-bridge pattern, or rely on emulator-only smoke + operator-driven full-sweep?

Each is a closed-set decision well-suited to AskUserQuestion if operator chooses to advance.

## Recommendation summary

Defer until v1.0.9 ships and operator demand is validated. Then a focused 3-week PWU lands Termux as a first-class third platform.

The technical risk is low. The maintenance commitment is real but bounded. The honest-gap citation is clean (we don't pretend Termux gives us what cgroups give us, and we explicitly document the Android-killer caveats per §11.4.6).

If at any point operator demand crystallises into a hard requirement ("this MUST work on Termux"), the work is well-understood and ready to begin.

## Sources

(synthesised from §01-§07; no new sources cited here — see prior files for evidence)